The University of Chicago ONR–604 Experiment Instrument Description

The high mass resolution needed for isotopic separation of heavy ions requires knowledge of the trajectory of each charged particle entering the acceptance cone of the telescope in order to determine the pathlengths in the detectors. As shown in figure 1(b), each position sensing detector (PSD) has a series of gold strips on the top surface of the detector which collect a fraction of the charge liberated by the passage of the particle. This signal is read out through a resistive divider network and compared to the total signal from the detector, measured by the charge collected on the back surface contact. The ratio of the two 1024 channel pulse height analyzers provide one coordinate of the point of incidence of the particle (Lamport, et al., 1976). Two sets of PSDs (three per set for redundancy) were used in the telescope to obtain the trajectory of the particle, or a total of 12 analyzers. Calibration of this system with accelerator particles has shown that the position can be measured to better than 0.1 mm: therefore, the angle of incidence of the particle could be determined to better than 1°.

The charge and mass of each particle was determined by the well-known technique of multiple dE/dx versus residual energy using the measurements provided by the stack of 5 mm thick lithium-drifted silicon detectors, corrected for the trajectory of the individual particle. For this analysis we required that the particle came to rest within the detector stack. This determines the energy range over which each type of particle can be studied. The signal from the scintillator guard counter was used in anti-coincidence mode to eliminate out-of-geometry events and to tag particles that exit from the side of the apparatus.

To separate isotopes for elements as heavy as nickel, it is necessary to measure the dE/dx and residual energy signals to a precision level of 0.1%. This stringent constraint has been met by developing exceptionally linear, highly stable electronic circuits to analyze the signals from each detector. Each analysis chain has been individually calibrated over a wide temperature range, and the entire instrument performance has been evaluated using high energy heavy ion beams obtained from the LBL Bevalac accelerator.

The performance of each analysis chain was monitored during flight by an on-board calibrator which periodically sent a sequence of 2000 pulses spanning the entire range of signal sizes. Furthermore, in-flight calibration and normalization was provided by measurement of galactic cosmic ray events. These high precision measurements required that the temperature remained stable and constant to within ~3 °C over each orbit: consequently the instrument power was on during 100% of the orbital period.

Because the anti-coincidence scintillator guard counter subtended a large solid angle and was used in the instrument logic, it was shielded from the high counting rates induced by low energy particles trapped in the magnetosphere and by intense solar flare fluxes. The shielding [shown in Figure 1(A)] corresponded to complete elimination of <4 MeV electrons by the AZ31B magnesium shield and the tantalum collar. The lower part of the scintillator was located well within the instrument package and was surrounded on five sides by electronic modules offering additional shielding. Backward moving particles were additionally shielded by the spacecraft structure.

Since about 50 percent of the time in each orbit was spent in the intense radiation portion of the magnetosphere (FIGURE 2) a 3mm external Al window shield was placed over the telescope in order to protect further against accumulated radiation damage to the detectors Dl, D2 and D3 (where the average geometrical factor was ~50 cm2-ster). This shield lowered the integrated proton and electron radiation doses for detectors Dl, D2, D3 while moving upwards only slightly the energy intervals for measurement of the heavy particles, which were the main objective of the experiment.

Figure 9 shows a functional block diagram of the instrument electronics. Besides providing pulse height analysis of each particle, the instrument had internal accumulators that counted and stored both single detector counting rates and coincidence counting rates, thus allowing the absolute sampling rate and the absolute flux levels to be deduced.

In addition to heavy particles the instrument measured protons and alpha particles over the energy range 25-119 MeV with a priority system to insure that the rare heavy ion events were preferentially analyzed and stored. In addition, in regions of expected high background radiation (i.e., the magnetosphere), a commandable heavy particle mode could be invoked. This raised the thresholds on detectors D1 through D6 to suppress the high firing rates induced by trapped protons and electrons while still permitting the rare heavy ions to be analyzed.

The following are examples of particle range/energy in the telescope:

|  | Detector | Minimum proton energy to reach detector* (MeV) | max (degrees) | Maximum proton energy to stop in detector** (MeV) |
| --- | --- | --- | --- | --- |
|  | D1 | 24.8 | — | 44.5 |
|  | D2 | 28.3 | 87 | 48.2 |
|  | D3 | 31.8 | 84 | 49.8 |
|  | D4 | 35.2 | 50 | 43.9 |
|  | D5 | 36.7 | 48 | 45.5 |
|  | D6 | 38.6 | 46 | 46.8 |
|  | K1 | 40.0 | 41 | 59.4 |
|  | K2 | 52.0 | 39 | 70.6 |
|  | K3 | 62.1 | 36 | 79.9 |
|  | K4 | 71.3 | 34 | 88.5 |
|  | K5 | 79.3 | 32 | 96.5 |
|  | K6 | 87.1 | 30 | 103.5 |
|  | K7 | 94.0 | 29 | 108.4 |
|  | K8 | 100.8 | 27 | 116.9 |

*normal incidence	**max angle of incidence

Examples of the geometrical factors for the telescope in FIGURE 1 are:

Logic *	Geometrical Factor (cm2-ster)

D1D2ND3D4N(S)	46.1

D13D46N(K1K2S)	10.9

D13K1K2N(K3S)	7.47

D13K1K8AN(S)	6.08

* D, K, A,S are detectors (see FIGURE 1). “N” indicates that all following detectors within the following parentheses are in anti-coincidence.

For further details on the instrument and the CRRES spacecraft, see Gussenhoven, Mullen, & Sagalyn (1985) and Simpson & Garcia-Munoz (1994).

APPENDIX 2: The Pathlength Distribution

The leaky-box equations of interstellar propagation are well-known. Under certain circumstances which are commonly assumed to be met, these equations are separable (details in Ginzburg & Syrovatskii 1964; Garcia-Munoz et al. 1987). The resulting “slab” equations can then be solved as a function of pathlength traversed and energy, while the other equation has a solution which is the Green’s function for the slab equations, namely, the pathlength distribution (PLD). This technique of solution is known as the “weighted-slab” method (Fichtel & Reames 1968). In the slab equations, spallation effects, energy loss due to ionization and radioactive decay are taken into account. The PLD is used to correct for the distribution of pathlengths traversed by the cosmic rays. The solutions of the coupled slab equations are number densities as a function of pathlength and energy Ni(x,E). These are integrated using a particular PLD(x,E) over the pathlength x to derive a local interstellar flux of nuclear species i.

The choice of the shape of the PLD used in our calculations is generally constrained by the requirement that the calculations agree with measured secondary/primary ratios. Garcia-Munoz et al. (1987) found below ~1 GeV/n that a pure exponential PLD with an energy-dependent escape length “tuned” to agree with the observed B/C data did not agree with the available experimental data on the (Sc-Mn)/Fe ratio. They showed that a simultaneous fit of both ratios could be obtained if an energy-dependent truncation of the PLD was used below ~1 GeV/n, and that the truncation increased with decreasing energy.

As described in SECTION 5, the PLD is required to fit measured secondary/primary elemental ratios. It has been found that while a single exponential PLD may agree with the Ulysses B/C and (Sc-Mn)/Fe ratios (DuVernois, Simpson, & Thayer, 1995), some slight truncation at low energies still appears necessary for agreement with all of the available data, including other recently published data (FIGURE 10). The truncation was modeled by the use of a double exponential (DE) PLD, as suggested by Simon (1977). The form of the DE PLD is:

This is properly normalized, and the energy dependence of the individual means x1(E) and x2(E) is chosen to agree with elemental secondary/primary ratios. FIGURE 11 shows the PLD means used to generate the curves in Fig. 10. It is important to note again that the particular choice of PLD (single exponential vs. double exponential or energy dependence of mean) has very little effect on the isotopic ratios, however. This can be seen by looking at the curves in FIGURES 6, 7 and 8. The solid lines in all these figures are the results of using a DE PLD which was used for previous CRRES source calculations (DuVernois et al., 1993; Garcia-Munoz et al., 1993), the dot-dashed lines are the result of using a DE PLD (FIGURE 11) with a much lower amount of truncation (controlled by mean x2(E)), and the short dashed lines (seen in FIGURES 6 and 8) are derived from using a single exponential PLD with the mean as determined in GM. The differences between the curves are in all cases <10%, and are certainly much smaller than the uncertainties in the fragmentation cross sections. It is clear that truncation of the PLD has a much smaller role in the question of determining isotopic ratios than may have been previously thought. The minor truncation required is in qualitative agreement with Webber (1993), who also found a slight truncation of the PLD at low energies necessary when performing a similar analysis.

BIBLIOGRAPHY

Anders, A., & Ebihara, M.    1982, Geochim. Cosmochim. Acta, 46, 2363

Anders, A., & Grevesse, N.    1989, Geochim. Cosmochim. Acta, 53, 197

Audouze, J.    1985, in ESO Proc. 21, Production and Distribution of C, N, O Elements, ed. I. J. Danziger, F. Matteucci, & K. Kjar (Garching: ESO), 373

Audouze, J., Cassé, M., Chieze, J.P., Malinie, J., & Paul, J. A.    1981, Proc. 17th Int’l. Cosmic Ray Conf. (Paris), 2, 296

Cassé, M., & Paul, J. A.    1982, ApJ, 258, 860

Cameron, A. G. W.    1982, in Essays in Nuclear Astrophysics, ed. C. A. Barnes, D. D. Clayton, & D. N. Schramm (Cambridge: Cambridge University Press), 23

Chenette, D. L. et al.    1994, IEEE Trans. Nucl. Sci., 41, 2332

Connell, J. J., & Simpson, J. A.    1993a, Proc. 23rd Int’l. Cosmic Ray Conf. (Calgary), 1, 547

Connell, J. J., & Simpson, J. A.    1993b, Proc. 23rd Int’l. Cosmic Ray Conf. (Calgary), 1, 559

Crane, P., Hegyi, D. J., & Lambert, D. L.    1991, ApJ, 378, 181

DuVernois, M. A., Garcia-Munoz, M., Pyle, K. R., & Simpson, J. A.    1993, Proc. 23rd Int’l. Cosmic Ray Conf. (Calgary), 1, 563

DuVernois, M., Simpson, J. A., & Thayer, M. R.    1995, Proc. 24th Int’l. Cosmic Ray Conf. (Rome), paper in session OG-5

Dwyer, R., & Meyer, P.    1987, ApJ, 322, 981

Engelmann, J. J., et al.    1990, A&A, 233, 96

Esposito, J. A., et al.    1992, Astropart. Phys., 1, 33

Ferrando, P., Lal, N., McDonald, F. B., & Webber, W. R.    1991, A&A, 247, 163

Fichtel, C. E., & Reames, D. V.    1968, Phys. Rev., 175, 1564

Fisk, L. A.    1971, J. Geophys. Res., 76, no. 1, 221

Garcia-Munoz, M., Simpson, J. A., & Wefel, J. P.    1979, ApJ Lett., 232, L95

Garcia-Munoz, M., Pyle, K. R., & Simpson, J. A. 1985, Proc. 19th Int’l. Cosmic Ray Conf. (La Jolla), 4, 409

Garcia-Munoz, M., Pyle, K. R., Simpson, J. A., & Thayer, M. R. 1993, Proc. 23rd Int’l. Cosmic Ray Conf. (Calgary), 1, 543

Garcia-Munoz, M., Simpson, J. A., Guzik, T. G., Wefel, J. P., & Margolis, S. H. 1987, ApJS, 64, 249 (GM)

Gibner, P. S., Mewaldt, R. A., Schindler, S. M., Stone, E. C., & Webber, W. R. 1992, ApJ Lett., 391, L89

Ginzburg, V. L., & Syrovatskii, S. I. 1964, The Origin of Cosmic Rays (New York: MacMillan)

Ginzburg, V. L., Khazan, Ya. M., & Ptuskin, V. S. 1980, Ap&SS, 68, 295

Gussenhoven, M. S., Mullen, E. G., & Sagalyn, R. C. 1985, CRRES/SPACERAD Instrument Description, Document AFGL-TR-85-0017, Air Force Geophysics Laboratory, Hanscom AFB, MA

Guzik, T. G., et al.    1990, Proc. 21st Int’l. Cosmic Ray Conf. (Adelaide), 3, 428

Guzik, T. G., et al.    1993, Proc. 23rd Int’l. Cosmic Ray Conf. (Calgary), 1, 555

Hsieh, K. C., & Simpson, J. A. 1970, ApJ Lett., 162, L191

Krombel, K. E., & Wiedenbeck, M. E.    1988, ApJ, 328, 940

Langer, W. D., & Penzias, A. A. 1993, ApJ, 408, 539

Lamport, J. E., Mason, G. M., Perkins, M. A., & Tuzzolino, A. J.    1976, Nuc. Inst. Meth., 134, 71

Leske, R. A.    1993, ApJ, 405, 567

Lukasiak, A., Ferrando, P., McDonald, F. B., & Webber, W. R.    1993, Proc. 23rd Int’l. Cosmic Ray Conf. (Calgary), 1, 539

Lukasiak, A., Ferrando, P., McDonald, F. B., & Webber, W. R.    1994, ApJ, 426, 366

Meyer, J. P.    1985, ApJS, 57, 173

Mewaldt, R. A.    1989, in Cosmic Abundances of Matter, ed. C. J. Waddington (AIP Conf. Proc. 183) (NY: AIP), 124

Moraal, H.    1976, Sp. Sci. Rev., 19, 845

Parker, E. N.    1965, Planet. Sp. Sci., 13, 9

Prantzos, N., Doom, C., Arnould, M., & de Loore, C.    1986, ApJ, 304, 695

Prantzos, N., Arnould, M., & Arcoragi, J.-P.    1987, ApJ, 315, 209

Sanderson, T. R., Marsden, R. G., Heras, A. M., Wenzel, K.-P., Anglin, J. D., Balogh, A., & Forsyth, R.    1992, Geophys. Res. Lett., 19, no. 12, 1263

Silberberg, R., & Tsao, C. H. 1990, Phys. Rep., 191, no. 6, 351

Simon, M.    1977, A&A, 61, 833

Simpson, J. A.    1983, Ann. Rev. Nucl. Part. Sci., 33, 323

Simpson, J. A., & Garcia-Munoz, M. 1988, Sp. Sci. Rev., 46, 205

Simpson, J. A., et al.    1992, A&AS, 93, 365

Simpson, J. A., & Garcia-Munoz, M.    1994, Final Report on USAF Contract No. F19628-90- 0025, Phillips Laboratory paper PL-TR-94-2189

Soutoul, A., Ferrando, P., & Webber, W. R.    1990, Proc. 21st Int’l. Cosmic Ray Conf. (Adelaide), 3, 37

Stahl, O., & Wilson, T. L. 1992, A&A, 254, 327

Thayer, M. R.    1995, Proc. 24th Int’l. Cosmic Ray Conf. (Rome), paper in session OG-8

Van Allen, J. A., & Fillius, R. W.    1992, Geophys. Res. Lett., 19, no. 14, 1423

Wannier, P. G., Andersson, B.-G., Olofsson, H., Ukita, N., & Young, K.    1991, ApJ, 380, 593

Webber, W. R.    1982, ApJ, 252, 386

Webber, W. R.    1993, ApJ, 402, 188

Webber, W. R., Kish, J. C., & Schrier, D. A.    1985, Proc. 19th Int’l. Cosmic Ray Conf. (La Jolla), 2, 88

Webber, W. R., Kish, J. C., & Schrier, D. A.    1990, Phys. Rev. C, 41, 566

Wiedenbeck, M. E., & Greiner, D. E.    1981a, Phys. Rev. Lett., 46, 682

Wiedenbeck, M. E., & Greiner, D. E.    1981b, ApJ Lett., 247, L119

Wilson, T. L., & Rood, R. T.    1994, ARA&A, 32, 196

Woosley, S. E., & Weaver, T. A. 1981, ApJ, 243, 651