OVERVIEW OF VOYAGER MAGNETOMETER EXPERIMENT

The magnetic field experiment carried onboard the Voyager 1 and 2 missions

consists of dual low field (LFM) and high field magnetometer (HFM) systems.

The dual systems provide greater reliability and, in the case of the LFMs,

permit the separation of spacecraft magnetic fields from the ambient fields.

Additional reliability is achieved through electronic redundancy.  The wide

dynamic ranges of +/- 0.5 G for the LFMs and =/- 20 G for the HFMs, low

quantization uncertainty of +/- 0.002 nT in the most sensitive +/- 8 nT

LFM range, low sensor RMS noise level of 0.006 nT, and use of data compaction

schemes to optimize the experiment information rate al combine to permit

the study of a broad spectrum of phenomena during the mission.  Objectives

include the study of planetary fields at Jupiter, Saturn, Uranus and Neptune;

satellites of these planets; solar wind and satellite interactions with

planetary fields; and the large-scale structure and microscale characteristics

of the interplanetary magnetic field.  The interstellar field may also be

measured.

Space Science Reviews, 21 (1977) 235-257.

DESCRIPTION OF VOYAGER MAG DATA SUBMISSIONS TO THE NSSDC

The high resolution data submitted to the NSSDC has its origins in the

original MVS "Summary" data sets formally produced on the NSSC IBM MVS

mainframe system.  These data sets contain a mix of engineering, electron,

magnetic field and plasma data.  The non-enginnering data contained

within these files consists of a set of averages applied across all of

the mission's differing telemetry modes.  In the case of the Magnetometer

Experiment, the records contain both 1.92 second, 9.6 second and 48 second

averages.  1.92 second averages are created from the detail detail data,

9.6 second averages are created from 1.92 second averages and 48 second

averages are created from the 9.6 second averages.  Thus, there are 25 1.92

second averages, 5 9.5 averages and one 48 second average per record.

In producing this submission to the NSSDC, the magnetic field records have

been stripped from the "Summary" files and converted from MVS to VAX

binary representation.  File content remains unchanged.  A second, ASCII

format data set containing key components of the 48 second magnetic field

data and ephemeris data has been created allowing more convenient access

to high resolution Voyager magnetometer data.

DATA FORMATS

Binary MAG data files contain two types of records.  The first and last

records are header records containing a description of the parameters

used to process the Experimental Data Records (EDRs).  The remaining

records contain the field averages from the low field magnetometers (LFM).

Each binary LFM record contains the following format:

Voyager Low Field Magnetometer Summary Record Format

Record Components

1.  Header block -     32 words

2.  MAG data block -  496 words

3.  SEDR data block -  40 words

---

568 words total (32 bit words)

Header Block Description

word type description

---- ---- -----------

1    A*4  Data Identifier ("HDR1" or "LFM ")

2    A*4  Telemetry Format ("ENG ","CR-1","CR-2","CR-3","CR-4","CR-5",

"CR-6","CR-7","CR5A","GS-2","GS-3","GS-4",

"GS-6","GS-7","GS-8","GS10","G10A","OC-1",

"OC-2","UV5A","****")

3    A*4  S/C Identification ("FLT1","FLT2","PTM ","SIM1","SIM2","UKWN")

4    I*2  Year of Data

4.5  I*2  Day of Year (Jan. 1 = 1)

5    I*2  Hour of Day

5.5  I*2  Minute of Hour

6    I*2  Second of Minute

6.5  I*2  Millisecond of Second

7    R*8  Decimal Day of Year (Jan. 1 = 0)

9    R*8  Decimal Day Count since August 20, 1977

11   A*4  Time Type (SCET = spacecraft event time,

ERTS = start earth receive time)

12   R*4  Time period of data block (48 sec)

13   I*2  48 Minute Counter (2**16 sequence counter of data at time

of telemetry readout.)

13.5 I*2  48 Second Counter (Modulo 60 sequence counter of data at time

of telemetry readout.)

14   I*2  60 Millisecond Counter (Line counter of data at time

of telemetry readout.)

14.5 I*2  Spare

15   Z*4  MAG Status Word

16   Z*4  MAG Command Word

17   I*2  Data Identifier (LFM=2,HDR1=8)

17.5 I*2  Data Rate (GS-X=0,CR-1=1,CR-2=2,CR-3=3,CR-4=4,CR-5=5,

CR-6=6,CR-7=9,ENG=8,CR5A=13,UV5A=13)

* CR5A and UV5A are both VIM-5 modes

18   I*4  Spare

19   A*8  Data Confidence Flag (indicates reduced data confidence due to

software/hardware intervention)

21   I*4  Spare

22   I*4  Spare

23   I*4  Spare

24   I*4  Spare

25   I*4  Spare

26   I*4  Spare

27   I*4  Spare

28   I*4  Spare

29   I*4  Spare

30   I*2  Number of primary data words

30.5 I*2  Number of secondary data words

31   I*2  Number of primary full words

31.5 I*2  Number of secondary full words

32   I*2  Record number in file

32.5 I*2  Number of words remaining in logical record

MAG Data Block Description

48 second averages

------------------

33   R*4  F1: 48 second average of 9.6 second norms (nT)

34   R*4  F2: Norm of 48 second component averages (nT)

35   R*4  Latitudinal field angle = arcsin(B1/F2) (degrees)

36   R*4  Longitudinal field angle = 180 - atan(B2,-B1) (degrees)

37   R*4  B1: 48 second average of first 9.6 second averaged component (nT)

38   R*4  B2: 48 second average of second 9.6 second averaged component (nT)

39   R*4  B3: 48 second average of third 9.6 second averaged component (nT)

40   R*4  RMS of B1 (nT)

41   R*4  RMS of B2 (nT)

42   R*4  RMS of B3 (nT)

43   I*4  Number of 9.6 averages per 48 second average (0<n<5)

9.6 second averages

-------------------

44-48  5(R*4)  F1: 9.6 second averages of 1.92 second norms (nT)

49-53  5(R*4)  F2: Norms of 9.6 second component averages (nT)

54-58  5(R*4)  Latitudinal field angle = arcsin(B1/F2) (degrees)

59-63  5(R*4)  Longitudinal field angle = 180 - atan(B2,-B1) (degrees)

64-68  5(R*4)  B1: 9.6 second average of first 1.92 second averaged component (nT)

69-73  5(R*4)  B2: 9.6 second average of second 1.92 second averaged component (nT)

74-78  5(R*4)  B3: 9.6 second average of third 1.92 second averaged component (nT)

79     R*4  RMS of B1 (nT)

80     R*4  RMS of B2 (nT)

81     R*4  RMS of B3 (nT)

.      .    .

.      .    .              5 sets of (B1,B2,B3) triplets

.      .    .

91     R*4  RMS of B1 (nT)

92     R*4  RMS of B2 (nT)

93     R*4  RMS of B3 (nT)

94-98  5(I*4)  Number of 1.92 averages per 9.6 second average (0<n<5)

1.92 second averages

--------------------

99-123  25(R*4)  F1: 1.92 second average of detail norms (nT)

124-148 25(R*4)  F2: Norm of 1.92 second component averages (nT)

149-173 25(R*4)  Latitudinal field angle = arcsin(B1/F2) (degrees)

174-198 25(R*4)  Longitudinal field angle = 180 - atan(B2,-B1) (degrees)

199-223 25(R*4)  B1: 1.92 second average of first detail component (nT)

224-248 25(R*4)  B2: 1.92 second average of second detail component (nT)

249-273 25(R*4)  B3: 1.92 second average of third detail component (nT)

274     R*4  RMS of B1 (nT)

275     R*4  RMS of B2 (nT)

276     R*4  RMS of B3 (nT)

.       .    .

.       .    .              25 sets of (B1,B2,B3) triplets

.       .    .

346     R*4  RMS of B1 (nT)

347     R*4  RMS of B2 (nT)

348     R*4  RMS of B3 (nT)

349-373 I*4  Number of detail points per 1.92 second average (0<n<32)

S/C field averages

------------------

374-520 Z*4  spare words

521     R*4  RMS of first 48 second averaged s/c field component (nT)

522     R*4  RMS of second 48 second averaged s/c field component (nT)

523     R*4  RMS of third 48 second averaged s/c field component (nT)

524     I*4  Number of 9.6 s/c field averages per 48 second average (0<n<5)

525     R*4  48 second average of first s/c field component (nT)

526     R*4  48 second average of second s/c field component (nT)

527     R*4  48 second average of third s/c field component (nT)

528     R*4  Norm of 48 second s/c field component averages (nT)

SEDR Data Block Description

---------------------------

529-530 R*8  TN:  Epic day of navigation block "closest" to data time

531-532 R*8  TP:  Epic day of pointing vector block "closest" to data time

533     R*4  s/c X position component (A.U. or planetary radii *)

534     R*4  s/c Y position component (A.U. or planetary radii *)

535     R*4  s/c Z position component (A.U. or planetary radii *)

536     R*4  s/c X velocity component (km/s)

537     R*4  s/c Y velocity component (km/s)

538     R*4  s/c Z velocity component (km/s)

539     R*4  s/c distance (A.U. or planetary radii *)

540     R*4  s/c longitude (radians *)

541     R*4  s/c latitiude (radians *)

542-550 R*4  Matrix 1

551-559 R*4  Matrix 2

560-568 R*4  Matrix 3

Notes:

* Spacecraft position data is dependent upon the coordinate system selected

during EDR processing.  The HDR1 record contains this information at word

number 13.  Valid coordinates are HG (heliographic), S3 (Jupiter planetary

system), L1 (Saturn planetary system), U1 (Uranus planetary system) and

N1 (Neptune planetary system).  During cruise processing (HG), the spacecraft

location is based upon the Inertial Heliographic system (IHG).  During

encounter processing (S3, L1, U1, N1), the spacecraft location is based upon

planocentric system coordinates.