PROCESSING VIM-5 TELEMETRY MODE MAG DATA

MACHINE ARCHITECTURE AND BIT REPRESENTATION

IBM and VAX machine architectures differ such that they are reversed

in their assignments of bit significance.  Data transfers between these

two machines must account for this difference to preserve word values.

Because EDR data is written for the machine architecture of an IBM 360,

proper interpretation of this data on a VMS system requires that each byte

of the LOGICAL*1 MAG science block be flipped.  Using an array of 256

LOGICAL*1 elements where each element indexed 0-255 is assigned the bit

flipped value of the index number, the 1040 bytes of the VIM-5 MAG

science block are bit flipped to render the proper bit order on the VAX.

UNPACKING VIM-5 HEADER BLOCK DATA

Voyager EDR header data contains many parameters reflecting the nature

of the engineering (ENG), magnetometer (MAG) and plasma (PLS) data that

follow it.  These descriptors include spacecraft id, record id, data mode,

time tags,  FDS counters, DSN parameters, data quality and data status.

While most fields are generic with respect to telemetry (data) mode, the

data presence indicators do vary in format with the use of differing modes.

UNPACKING VIM-5 SUB-HEADER BLOCK (PLS) DATA (SUBROUTINE PLSVIM5)

The subroutine PLSVIM5 serves to unpack MAG and PLS command words, MAG

status words, starting spacecraft event time of day and PLS data words.

MAG command words provide information about the following commanded MAG

parameters:

WORD 1                                WORD 2

IBHFM Calibrate State                 OBHFM Calibrate State

IBHFM Range State                     OBHFM Range State

IBHFM Auto Ranging State              OBHFM Auto Ranging State

IBHFM Clock State                     OBHFM Clock State

Electric Flipper State                Prime MAG Selection

IBLFM Calibrate State                 OBLFM Calibrate State

IBLFM Range State                     OBLFM Range State

IBLFM Auto Ranging State              OBLFM Auto Ranging State

PLS command words provide information about the following commanded PLS

parameters:

Mode

Amplifier Gain

Integrator gain

Modulator Voltage Monitor

Current Calibration

IDC Measurement

Modulator

MAG status words are most important to the processing of Voyager EDR's.

MAG status words contain information about the actual configuration of

the magnetometers as dictated by the MAG command words.  MAG status words

yield the following values:

WORD 1                                WORD 2

OBLFM Range Mode                      OBLFM Flight Calibration

OBLFM Range                           IBLFM Flight Calibration

IBLFM Range Mode                      OBHFM Flight Calibration

IBLFM Range                           IBHFM Flight Calibration

OBHFM Range Mode                      IFC Polarity

OBHFM Range                           Processor Selection

IBHFM Range Mode                      IBLFM Flipper Status

IBHFM Range                           IBLFM Flipper Position

IBLFM Flipper Status

IBLFM Flipper Position

Prime LFM Status

Electrical Flipper

UNPACKING VIM-5 MAG COUNTS (SUBROUTINE MAGVIM5)

The subroutine MAGVIM5 serves to unpack the raw magnetometer counts

from the VIM-5 science block as described in the document ECR #37734.

In accordance with the above data description, tables of bit offsets

for the 12 bit Primary and Secondary Reference Words and the 6 bit

Primary Difference Words for each magnetometer axis are placed in

data statements at the top of the MAGVIM5 subroutine.  Reference Word

tables contain one offset value for each five minor frame cycle that

occurs in the twenty minor frame VIM-5 MAG science block.  An offset

value of negative one indicates that no Reference Word is present for

the associated five minor frame cycle.   Difference Word tables contain

up to twenty five offset values for each five minor frame cycle.

Similarly, an offset value of negative one indicates a null occurrence

of a Primary Difference Word.  One needs to note that while document

ECR #37734 describes a 416 bit minor frame indexed by bits 1-416, the

FORTRAN 77 bit referencing module (MOVBIT) employed by the VIM-5

processing software indexes these bits as 0-415.  This is reflected in

the starting bit values of the offset tables.

Reference and Difference Words are laid out such that the first bit

encountered of each word is the most significant bit.  Reference Words

are 12 bits in length and are unsigned integers.  Therefore bits 0-11

are mapped into bits 11-0 of an INTEGER*2 word.  Difference Words are 6

bits in length and are signed integers.  Therefore bits 0-5 are mapped

into bits 5-0 of an INTEGER*2 word and to preserve sign, values greater

than 31, the maximum value represented by the 5 non-sign bits, have the

value of 64 subtracted from them to render the original signed value.

This sign preservation method is faster than assigning the sign bit to

bit 11 and propagating the sign bit value through bits 10-5 of the

INTEGER*2 word (GS-3 technique).

PRODUCING VIM-5 LFM GAMMAS (SUBROUTINE VIM5LFM)

The subroutine VIM5LFM serves to call routines that extract instrument

status parameters from MAG status words, reconstruct Primary Full Words

from Primary Difference Words, perform sensor and boom alignment

rotations, and convert digital VIM-5 LFM counts into units of gammas.

EXTRACTING MAGNETOMETER RANGE AND MODE VALUES (SUBROUTINE VIM5COMM)

MAG status words unpacked from the plasma subheader contain range and

mode values for the inboard and outboard low field magnetometers. High

field magnetometers are not utilized in the VIM-5 mode.  The reference

article entitled Voyager Conjoint Magnetic Field and Plasma Science Data

Management System describes the contents of the two MAG status words.

Each five minor frame cycle of the VIM-5 MAG science block is referenced

by both MAG status words, STAT1 and STAT2.  Three bit range values

describing the eight sensitivity ranges (0-7) are extracted from the MAG

STAT1 word in reverse bit order where bits 0-2 are mapped into bits 2-0 of

an  INTEGER*2 word.  One bit mode values specifying range step control are

also extracted from the MAG STAT1 word and are mapped into an INTEGER*2

word at bit 0.  The MAG STAT2 word contains information pertaining to

the state of the sensors and supporting electronics.

RECONSTRUCTION OF 12 BIT PRIMARY FULL VECTOR WORDS (SUBROUTINE VIM5DIFF)

Primary Full Vector Words are reconstructed by adding the accumulated

sum of Primary Difference Words to the last previously encountered

Primary Reference Word.  When a new Primary Reference Word is

encountered, the calculated Primary Full Vector Word is compared to the

Primary Reference Word.  If the two values are not equal, then an error

has occurred somewhere in the reconstruction of the last 25 (VIM-5 mode

only) Primary Full Vector Words and an error flag marks this event.

Since there is no way of pinpointing where a reconstruction error has

occurred between Primary Reference Words, all 25 reconstructed Primary

Full Vector Words need be removed.  After comparing a Primary Full

Vector Word to a new Primary Reference Word, the Primary Difference

Word accumulator is reinitialized to zero.

SENSOR ALIGNMENT ROTATION

Sensor alignment is achieved by passing a 3x3 sensor matrix and raw

counts to SUBROUTINE MPRD31 which perfoms the matrix multiplication

for rotation.

BOOM ALIGNMENT ROTATION

Boom alignment is achieved by passing a 3x3 boom matrix and sensor

rotated counts to SUBROUTINE MPRD31 which perfoms the matrix

multiplication for rotation.

CONVERSION OF DIGITAL VALUES INTO GAMMAS (SUBROUTINE VIM5CNT)

Reconstructed Primary Full Vector Words and Secondary Reference Words

are converted into gammas by subtracting the appropriate zero offset

and multiplying this difference by the sensitivity of the sensor.  Zero

offsets and sensitivities are obtained from precalculated tables.

Range and offset words index the correct line of the offset and

sensitivity table to get the required values.  If one or more axes of

a magnetometer render null digital values (0), then all axes and the

magnitude are assigned a gamma fill value of 999.

DATA FILTERING (OPTIONAL)

Before averages are computed for magnetometer data, there is an option

to employ A. Robert's population data filter to remove noise points from

the detail data.  Data points and time tags are buffered by SUBROUTINE QUE

(approx. 50000 data points) to allow a more efficient employment of the filter.

1.92 SECOND AVERAGING OF SECONDARY DATA (SUBROUTINE VIM5SEC)

To allow production of averages of higher resolution than the secondary

magnetometer sampling rate (24 sec in VIM-5 mode), secondary detail data

is interpolated to yield a sampling rate equal to that of the primary

magnetometer (.48 sec).  Interpolation consists of a simple stretching

of values between samples to yield a step function.  A true linear

interpolation is not performed.

1.92 SECOND AVERAGING PRIMARY MAG DATA (SUBROUTINE VIM5PRI)

To produce the 1.92 second averages required for input into the summary

output routine BIGAVE, all axes and the magnitude of the primary

magnetometer are averaged over 1.92 second intervals.  Counts per interval

are returned with interval averages.

AVERAGING SUMMARY DATA (SUBROUTINE BIGAVE)

The subroutine BIGAVE serves to create the 9.6 second and 48 second

averages of the primary and secondary MAG data streams that are needed

for the Summary Data Format.  These averages are created from an inputted

set of 25 1.92 second averages.

AVERAGING

To produce Summary magnetometer data, the three 1.92 second field

components averages and the corresponding 1.92 second field magnitude

of each magnetometer are averaged over five 9.6 second intervals and

then averaged into one 48 second interval.  Counts per interval are

returned with interval averages.