VOYAGER MAGNETOMETER EDR PROCESSING SOFTWARE

Last Revision:  03/04/97

********************************************************************************

SYSTEM OVERVIEW

The purpose of the Voyager Magnetometer EDR Production System (VOYPROD) is

to take raw experiment data from the Voyagers' magnetometers, decode it, and

perform limited data analyses producing high quality summary or detail data

that may be used for display or extended analysis.  The VOYPROD software

was developed on a DEC VMS platform using the FORTRAN 77 language.  No

assembler routines were employed in the extraction, unpacking and

reconstruction of magnetometer (MAG) data.  Unlike the original MVS-based

processing system, the Voyager Conjoint Magnetic Field and Plasma Science

Data Management System, VOYPROD deals exclusively with magnetometer and

engineering data.  Plasma processing is performed separately by MIT.

The principle inputs to the VOYPROD system are Experimental Data Records

(EDRs) which contain both science and engineering telemetry records, and

Supplementary Experimental Data Records (SEDRs) which contain spacecraft

navigation and pointing vector records.  The VOYPROD system processes

Voyager MAG EDRs and SEDRs to produce either a Detail Data Product

containing field data of the highest available resolution or a Summary Data

Product containing various averages (1.92, 9.6 and 48 second) of detail

field data.  VOYPROD also unpacks engineering records and outputs engineering

measurements in a format easily intepreted by post processing analysis

programs.

********************************************************************************

SYSTEM OPERATION

VOYPROD decommutates Voyager MAG EDRs to obtain integer magnetometer counts

which are converted to units of magnetic field strength (gammas) by

subtracting an offset value (zeros) and multiplying the resulting difference

by the sensitivity value of the instrument as provided by the MAG status

word.  When selected, VOYPROD uses SEDRs to provide the navigation and

pointing vector information necessary to transform decommutated magnetic

field data from the onboard payload coordinate system into either heliographic

coordinates (RTN) or planetary coordinates (S3=Jupiter, L1=Saturn, U1=Uranus,

N1=Neptune).  Coordinate transformations are applied to either detail data or

1.92 second averages depending upon whether the Detail Data Product or the

Summary Data Product is produced.  VOYPROD provides an option to filter data

at the detail level using a population filter to eliminate noise spikes.

********************************************************************************

PROCESSING OUTLINE (MAG science data)

-> Read EDR

-> Unpack header block (rec. id, s/c id, tel. mode, FDS counts, data flags)

-> Convert selected time tags to integer time (yy/ddd/hh:mm:ss.fff)

-> Unpack sub-header block  (MAG status words, plasma data)

-> Unpack science block (MAG counts)

-> Convert counts to gammas

-> Apply sensor and boom alignment matrices

-> Filter detail data based on standard deviation (optional)

-> Rotate (optional) detail gammas (Detail Data only) OR

rotate (optional) 1.92 second averages while averaging detail gammas

to form 1.92, 9.6 and 48 second averages (Summary only)

-> Write Summary or Detail record

********************************************************************************

CALLING SEQUENCE

MAIN       main routine

UNHEAD     unpack header block

UNPACKXXX  unpack sub-header and science blocks

PLSXXX     extracts MAG status and PLS words from sub-header block

MAGXXX     extracts MAG words from science block

XXXLFM     make gammas

MAGSTATUS  extracts instrument range and state

* DIFFERENCE restores full words from differenced words (GS-3, CR-4, VIM-5)

* DELTA      restores full words from delta modulated words (CR-5, CR-6)

* SHIVOT     pivot and shift reconstructed delta words to correct

DC level (CR-5, CR-6 only)

MAKEGAMMAS subtracts zero offset from counts and multiples resulting

difference by the instrument's sensitivity

QUE        buffer data for population filtering

XXXAVE     make summary averages or output detail files

XXXFLD     calculates ambient and spacecraft fields

XXXDET     writes detail data

XXXPRI     averages primary MAG data

XXXSEC     averages secondary MAG data

XXXPNS     substitutes secondary MAG data for spacecraft field

data to allow output of separate primary and secondary

MAG averages in the same summary record (all except CR-1)

********************************************************************************

MAG DATA

MAG data from all telemetry modes is processed through four top level

sets of routines, UNPACKXXX, MAGXXX, XXXLFM, and XXXAVE, where XXX

represents the telemetry mode being processed.  The routine UNPACKXXX

separates MAG science data and PLS subheader data from the the MAG EDR.

The PLS subheader data block is passed to the routine PLSXXX where plasma

words and crucial MAG status words are unpacked.  The MAG data block is

passed to the routine MAGXXX.  The MAGXXX routine unpacks MAG integer words

from the science data stream.  MAG words consist of 12 bit MAG full words

(all modes) and 2 bit delta modulated (modes CR-5 and CR-6) or 6

bit difference words (modes GS-3, CR-4, and VIM-5).  Routine XXXLFM calls

subroutines to extract status words unpacked from subheader data, reconstruct

delta modulated or differenced words where applicable, convert integer MAG

counts into units of magnetic field strength (gammas) and apply sensor and

boom alignment corrections to the MAG field data.  The routine XXXAVE calls

subroutines to compute spacecraft and ambient fields, and either output

detail data or create a series of averages for output as summary data.

********************************************************************************

TELEMETRY MODES

VOYPROD is currently capable of processing EDR's containing the following

telemetry modes:

GS-X  (encounter and cruise modes)

CR-1  (cruise mode)

CR-2  (cruise mode)

CR-3  (cruise mode)

CR-4  (cruise mode)

CR-5  (cruise mode)

CR-6  (cruise mode)

VIM-5 (cruise mode)

ENG   (engineering data)

This represents all telemetry modes used throughout the mission to date.

********************************************************************************

EDR FORMAT

The EDR files used by the Voyager Magnetometer Project contain two types of

records, engineering records (ENG) and magnetic field records (MAG).

The Voyager MAG EDR is composed of three major blocks, the header, the

sub-header or plasma block, and the science block containing MAG counts.

Header Block

The header block is a generic descriptor block that provides basic information

necessary to properly interpret the science and engineering data that follows

it.  Spacecraft identification, time tags, telemetry configuration, data status

and quality are some of the parameters contained within the header block.

Most fields within the header block are statically defined for the whole of the

mission.  An exception to this rule are the fields indicating data presence.

These fields change size and format between modes.  Both MAG and ENG EDRs

use a standard 240 byte header block.

Sub-Header Block

The ENG EDR sub-header block is an empty data block containing fill words.

The MAG EDR sub-header block contains MAG and plasma (PLS) command words,

the MAG status words, and PLS data.  MAG and PLS command words reflect the

most recently commanded instrument states.

(MAG and PLS command description)

The MAG status words provide information regarding operating state of the MAG

instruments. These parameters include range, mode and calibration settings.

Values extracted from MAG status words are used in production to map the

correct sensitivity range to digital magnetometer values.

(MAG status description)

Plasma data consist of eight bit words.  They are unpacked from the sub-

header block but are not employed in the MAG science processing.

Science Block

The ENG EDR contains measurements from 243 engineering parameters.

Measurements are sampled from a 60 word deck that is sampled 60 times per

EDR.

The MAG EDR science block contains digital magnetometer counts.  Counts are

measured onboard using 12 bit words that may represent values ranging from

0-4096.  Counts are formatted using a number of differring methods dependent

upon the telemetry mode employed.  There are four format types used for

recording MAG science data; direct readout, time averaged, differenced and

delta modulated.

The Voyager ENG EDR is composed of three major blocks, the header, the

sub-header and the engineering science block containing ENG measurements.

********************************************************************************

DATA RECONSTRUCTION

Four types of data formatting schemes were used to store magnetometer

counts; direct readout, time averaging, 6-bit differencing and adaptive

2-bit delta modulation.  Each succeeding formatting scheme provides an

increasing level of data compaction necessary as telemetry bandwidth

decreases with the continual increase in spacecraft distance from the

Earth.

Direct Readout

Direct formatting is the simplest method of representing MAG science data.

The direct format type contains a stream of 12 bit full word MAG counts

that are representative of the data rate of the MAG instrument itself.

The MAG instrument has a .06 second per sample data rate and thus direct

readout data provides this level of resolution.  The CR-1 mode is the only

mode employing the direct telemetry format.

Averaging

Averaged formatting is similar to the direct format.  Detail data from the MAG

instrument is averaged over a mode dependent interval then written as a 12 bit

full word.  The CR-2 and CR-3 modes employ averaged telemetry formatting.

Differenced

The differencing scheme compacts the data stream by placing a number of 6 bit

difference counts in between 12 bit reference full words.  The 6 bit difference

words represent the arithmetic difference between a current MAG count and the

prior count from the same instrument axis.  Full words are reconstructed by

adding accumulated difference words to the last full reference word encountered.

The GS-3, CR-4 and VIM-5 modes employ difference formatting.  The subroutine

XXXDIFF restore 12 bit full words from 6 bit differenced words.

Delta modulation

The delta modulation scheme compacts the data stream by placing a number of

2 bit delta modulated words in between 12 bit reference full words.  The 2

bit delta modulated words can be reconstructed to form 12 bit predicted

values which are then fitted to the level of the reference words using a

shift and pivot method.  The CR-5 and CR-6 modes employ the delta modulation

scheme of telemetry formatting.  The CR-6 telemetry mode was flawed in its

creation of delta modulated words.  Valid data may be obtained by processing

CR-6 data using only the 12 bit reference words.  The subroutines DELTA and

SHIVOT reconstruct 12 bit full words from compacted 2 bit delta modulated

words.

********************************************************************************

INPUT PARAMETERS

The Voyager Magnetometer EDR Processing Program, VOYPROD, requires the following

input as ordered:

1.  Log file name

2.  Navigation file name

3.  Pointing Vector file name

4.  Zeroes table file name

5.  Sensitivities file name

6.  Sensor alignment file name

7.  Boom alignment file name

8.  EDR input file name

9.  Output file name (detail or summary, depending upon processing)

10.  MAG system flags vector (see description to follow)

11.  EDR processing start time (yy/ddd/hh:mm:ss.sss)

12.  EDR processing stop time (yy/ddd/hh:mm:ss.sss)

13.  Time format  (SCET=spacecraft event time,

ERTS=Earth receive start time

ERTE=Earth receive end time)

14.  One way light time constant (minimum delta t / t for adjacent records)

15.  Range value override (-1=disable, 0-7=forced value)

16.  Inboard initial range value (0-7)

17.  Outboard initial range value (0-7)

18.  Mode value overide (-1=disable, 0=manual, 1=auto)

Auto mode allows range to change by one (1) between adjacent status

words.

19.  Inboard initial mode value (F=manual, T=auto)

* F recommended to allow recovery from improper initial range value.

20.  Outboard initial mode value (F=manual, T=auto)

* F recommended to allow recovery from improper initial range value.

21.  Primary mode initial value (F=outboard prime, T=inboard prime)

22.  Population filter limit (real value)

23.  Population filter passes (integer value)

********************************************************************************

MAG PROCESSING CONTROL FLAGS

Voyprod uses the following flags vector to control EDR processing:

SYS2(1)  WRITE 1.92 SECOND AVERAGES

SYS2(2)  WRITE 48.0 SECOND AVERAGES

SYS2(3)  PROCESS ENGINEERING RECORDS

SYS2(4)  WRITE MAG DETAIL DATA PRI, SEC PAYLOAD

SYS2(5)  WRITE MAG DETAIL DATA AMB, S/C FIELD PAYLOAD

SYS2(6)  WRITE DETAIL DATA

SYS2(7)  ROTATE DETAIL DATA

SYS2(8)  ENABLE DELTA MODULATION RECONSTRUCTION

SYS2(9)  SUPPRESS COORDINATE SYSTEM ROTATION

SYS2(10) SUPPRESS DATA PRESENCE CHECK

SYS2(11) SUPPRESS DATA QUALITY CHECK

SYS2(12) ENABLE POPULATION FILTER

SYS2(13) BYPASS SENSOR ALIGNMENT

SYS2(14) BYPASS TIME PROGRESSION CHECK

SYS2(15) FORCE ZEROS TO FIRST TABLE

SYS2(16) BYPASS ONE WAY LIGHT TIME CHECK

SYS2(17) JUPITER PLANETARY COORDINATES

SYS2(18) SATURN PLANETARY COORDINATES

SYS2(19) URANUS PLANETARY COORDINATES

SYS2(20) NEPTUNE PLANETARY COORDINATES

SYS2(21) PRIMARY MAG ONLY

SYS2(22) SECONDARY MAG ONLY

SYS2(23) BYPASS BOOM ALIGNMENT

SYS2(24) FORCE IB MAG AS PRIMARY

SYS2(25) WRITE MAG/PLS COMMAND WORDS AND MAG STAT WORDS

SYS2(26)

SYS2(27) PROCESS S/C FIELD AS AMBIENT

SYS2(28)

SYS2(29) OVERRIDE RECORD CHECK ROUTINE

SYS2(30)

SYS2(31) WRITE COUNTS

SYS2(32) WRITE GAMMAS BEFORE SENSOR AND BOOM ALIGNMENT

********************************************************************************